CP502 Advanced Fluid Mechanics (30-hr Project)




                           September 2010

Project 4: Unsteady-state motion-driven flow between two parallel plates
4.1 
A stationary incompressible Newtonian fluid is contained in the space between two infinitely wide parallel plates placed at a distance B apart from each other. At time t = 0, the lower plate is set in motion at velocity V in x-direction and the fluid begins to flow. Assuming one-dimensional flow, show that the flow velocity (ux) is governed by the following equation: 


[image: image1.wmf]2

2

y

u

t

u

x

x

¶

¶

=

¶

¶

m

r

       







(4.1)
where ρ is the density, μ is the viscosity and y is the distance perpendicular to the plates measured from the lower plate. 

4.2 
Reason out the initial and the boundary conditions given below: 


[image: image2.wmf]0

)

,

0

(

=

=

y

t

u

x

;

[image: image3.wmf]V

y

t

u

x

=

=

)

0

,

(

;
     
[image: image4.wmf]0

)

,

(

=

=

B

y

t

u

x



(4.2a, b and c)

4.3
Show that the steady-state velocity profile and the corresponding volumetric flow rate per unit plate width are given by the following equations: 


[image: image5.wmf]÷

ø

ö

ç

è

æ

-

=

B

y

V

u

steady

x

1









(4.3)

[image: image6.wmf]2

BV

Q

steady

=










(4.4)
4.4 
Using the non-dimensionless quantities, 



[image: image7.wmf]V

u

U

x

=

, 

[image: image8.wmf]B

y

=

h



and

[image: image9.wmf]2

B

t

r

m

t

=

,



(4.5a, b and c)

reduce equations (4.1) and (4.2) to the following: 


[image: image10.wmf]2

2

h

t

¶

¶

=

¶

¶

U

U










(4.6)

[image: image11.wmf]0

)

,

0

(

=

=

h

t

U

;

[image: image12.wmf]1

)

0

,

(

=

=

h

t

U

;
 
[image: image13.wmf]0

)

1

,

(

=

=

h

t

U



(4.7a, b and c)

4.5
Solve equations (4.6) and (4.7) using appropriate numerical methods.

4.6
Simulate the velocity profile ux with time t for different values of V and show that it approaches the steady velocity profile as t goes to infinity. Take the fluid to be water at 27oC and the distance between the plates B as 0.1 m.
4.7
Plot the variation in the volumetric flow rate per unit plate width as a function of time, and show that the volumetric flow rate approaches the steady volumetric flow rate as t goes to infinity.

Reference: Ismail Tosun, Modelling in Transport Phenomena: a conceptual approach. Second edition 2007, Elsevier.     



























PAGE  

_1347050975.unknown

_1347050998.unknown

_1347051196.unknown

_1347051258.unknown

_1347051289.unknown

_1347051247.unknown

_1347051078.unknown

_1347050983.unknown

_1346965131.unknown

_1346965165.unknown

_1347046485.unknown

_1346964859.unknown

_1346958871.unknown

